# Week 4 – Tutorial Questions

## Option 1 – Continue with week 1’s Simscape

If you didn’t get round to finishing last sessions simscape exercise, feel free to keep working on this.

## Option 2 – Simulate and optimise a sealed cabinet loudspeaker frequency response

A sealed cabinet loudspeaker can be modelled as a mass on a spring system. The driver/cone acts as the mass, the suspension/spider provides the spring stiffness and damping. The volume of air within the cabinet acts as a second spring in parallel with the suspension stiffness.

The radiated sound pressure from a loudspeaker is proportional to its acceleration. We can estimate the frequency response of the loudspeaker (not including sensitivity) by calculating the accelerance FRF of the mass spring system.

- Go online and find a loudspeaker driver with its corresponding spec sheet. Or use the driver below,

| Parameter | Value |
| --- | --- |
| Resonant frequency | 119.8 Hz |
| Diaphragm Mass Inc. Airload (Mms) | 2.2 g |
| Mechanical Compliance of Suspension (Cms) | 0.79 mm/N |
| Mechanical Q (Qms) | 7.33 |
| Surface area (S) | 28.3 cm² |

- Verify the resonant frequency using the supplied mass and compliance values

- Convert the Q-factor damping into a viscous damping coefficient c using the equation,

QMS=1cMMSCMS

and then into the viscous damping factor ζ.

- Calculate and plot the accelerance of the driver with no added cabinet.

- Calculate the equivalent stiffness of a cabinet with a volume of your choosing.

- Calculate and plot the accelerance of the driver with an added cabinet (you should use a parallel spring)

- Play around with the cabinet volume to obtain a ‘nice’ frequency response.

- You might also try to add some cabinet damping… How does this effect the loudspeaker response?

## Option 3 – Bottoms up!

We have seen that the humble beer/wine/gin bottle is in-fact a mass-spring assembly. The plug of air residing in the neck of the bottle acts as a mass. The volume of air in the bottle acts as a spring (with some damping). Let’s explore this further with some simple experiments…

You will need: at least one empty bottle (preferable the bottle should have a relatively sharp transition between its neck and the main volume), ideally a second bottle with the same volume, but a different size neck, a measuring jug, and a means of measuring the acoustic response (I use the phone app ‘Decibel X’.

### Part 1 – Validate form of equation

We have seen that the resonant frequency of a system is equal to ω=km. The mass of the neck is simply m= ρ0LS. The stiffness related to volume by, k=ρ0c2S2V, with V being the volume in m^3. We can begin to verify this equation by comparing the resonant frequency of the bottle with two different volumes.

- Measure the resonant frequency of the empty bottle

- Measure the resonant frequency of the half full bottle

- How does the frequency change relative to the volume? Does this agree with the equation?

- How might you improve the estimate of this relation?

- If you do manage to find two bottles, one with a different neck, try repeating this process using the different neck masses.

### Part 2 – Estimate the resonant frequency of your bottles

- Using the volume and neck geometry, estimate the resonant frequency of your bottles. How do the results compare against measurement?

- What might the differences be caused by?